Suspension Geometry Program - written by Wm. C. Mitchell

(c) Copyright 1983, 1997 - All rights reserved.

Body/CAD program version 6.00;    January, 1997.

The file name is SRF_FRT.RSF

*********************************************************

SRF Front w/3 1/8" Ride Height (No Driver, No Body)

Spec Racer Front Suspension

*********************************************************

Suspension type is 3 using SRF_F97.GEO for Rocker Arm

MacPherson_Type is False

Solid Rear axle type is False

Watts-pivot Type is False

Steering box/drag link type is False

Double Crank (Lola-Indy Car Rear) is False

ROCKER Structure specification:    Double A-Arm with Swaybar

Upper and lower wishbones with a steering tie-rod.

Model supports spring/shock, swaybar and halfshaft but not bellcrank.

Spring/shock and swaybar may be attached to:    Lower wishbone

Upper wishbone (default)

Hub

The steering Tie-rod may be attached to:    Tub, chassis or frame (default)

Lower wishbone

Upper wishbone

Specify Suspension and Attachments:

Steering tie-rod attached to Tub

Do NOT process half-shaft

Spring/Shock absorber attached to Upper

Perform extra Motion Ratio computations (adds processing time).

Anti-roll or stabilizer bar attached to Upper

NO Push/Pull rod:    Spring/Shock absorber operates directly

SRF Front w/3 1/8" Ride Height

Suspension Points: Both sides symmetric      Fore/Aft              Width        Vertical

Coordinate system origin is at                    0.000              0.000              0.000

Lower-inner forward tub mount      A            -4.500              9.750              5.500

Lower ball joint                                B            -0.250            26.500              6.750

Lower-inner rearward tub mount    C              9.500              9.750              6.187

Upper-inner forward tub mount      D            -0.500            13.000            15.750

Upper ball joint                                E              0.000            22.500            17.000

Upper-inner rearward tub mount    F              6.250            13.000            14.750

Steering Arm mount on hub              S            -4.750            25.250            10.000

Tub mount for steering arm            T            -3.750              9.750              8.500

Wheelbase and Track                                        92.000            56.750

Rim Width and Tire Diameter                                                  7.000            21.000

Initial Camber and Toe settings                -5.000              0.000

Center of Gravity                                          -35.750                                    12.780

Toe Measure/Steering Rack Travel                                        0.000

Tub mount of spring/shock              U              5.500              1.750              3.875

Activation end of spring/shock    V              5.250              4.750            17.250

Roll-bar blade pivot point            X            -0.250              6.500              6.250

Roll-bar mount inboard                    Y            -0.250              0.000              6.250

Roll-bar blade meets drop-link    Z              4.625              6.500              6.250

Roll-bar drop-link attachment      R              4.500              6.500            15.500

Chassis clearance check                  L              0.000              0.000              0.000

SRF Front w/3 1/8" Ride Height

Computed suspension lengths: Both sides

Length of forward Lower link      A-B            17.326

Length of rearward lower link    C-B            19.389

Perpendicular lower link length                  16.782

Front View Projection lower length            16.782

Length of forward Upper link      D-E              9.595

Length of rearward upper link    F-E            11.592

Perpendicular upper link length                    9.713

Front View Projection upper length              9.592

Distance between ball joints      B-E            11.006

Tie Rod Length                                  S-T            15.604

Rollout or tire circumference                      65.973

Damper Length                                    U-V            13.710

Swaybar Blade length                      X-Z              4.875

Swaybar Rod length                          Z-R              9.251

Static Caster (in degrees)                              1.397

King Pin angle (in degrees)                          21.318

King Pin Offset/Scrub Radius                        -0.759

Caster Trail                                                          0.415

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is    0.000 (Droop or Rebound)          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -5.000        -5.000              0.000          -0.387          -0.003      -0.003    -0.007

1.00          -5.826        -4.229            -2.961          -0.386            0.001      -0.008    -0.007

2.00          -6.709        -3.510            -6.030          -0.383            0.006      -0.012    -0.006

3.00          -7.653        -2.844            -9.286          -0.376            0.011      -0.017    -0.006

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00            1.397          1.397          0.000      -0.000    -0.000              55.470            9.601

1.00            0.872          1.912          0.032        0.031    -0.000              56.666            9.699

2.00            0.334          2.417          0.063        0.062    -0.001              57.861            9.791

3.00          -0.216          2.912          0.093        0.090    -0.003              59.072            9.878

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00            141.722          -1.932            141.722          -1.932              -0.387          13.167

1.00            195.195          -2.965            112.033          -1.381              -0.386          13.166

2.00            322.916          -5.538              92.960          -1.036              -0.383          13.163

3.00          1065.434        -20.981              79.548          -0.795              -0.376          13.156

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00            0.000          0.000            0.862          0.861          0.742          0.742        -0.000

1.00            0.428        -0.426            0.866          0.857          0.749          0.735        -7.623

2.00            0.858        -0.849            0.870          0.853          0.757          0.728      -15.282

3.00            1.290        -1.269            0.875          0.847          0.766          0.717      -23.015

SRF Front w/3 1/8" Ride Height

Ride Travel is    0.000 (Droop or Rebound)          Steering Rack travel is      0.000

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.862          0.861            0.00%          0.00%          Plunge not done

1.00            0.866          0.857            0.95%        -0.94%          Plunge not done

2.00            0.870          0.853            1.98%        -1.88%          Plunge not done

3.00            0.875          0.847            3.19%        -3.27%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00            0.404          0.404            0.800          0.800

1.00            0.294          0.511            0.855          0.747

2.00            0.177          0.616            0.912          0.695

3.00            0.054          0.720            0.973          0.643

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                -0.000

1.00                  0.001

2.00                  0.003

3.00                  0.005

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is    1.000 (Droop or Rebound)          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -4.707        -4.707              0.000            0.512          -0.006      -0.006    -0.012

1.00          -5.652        -3.823              3.624            0.495            0.001      -0.012    -0.011

2.00          -6.662        -2.996              8.001            0.440            0.009      -0.018    -0.010

3.00          -7.745        -2.224            14.318            0.322            0.017      -0.024    -0.007

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00            0.320          0.320          0.063      -0.063    -0.127              52.921            9.367

1.00          -0.223          0.850          0.094      -0.032    -0.126              54.139            9.477

2.00          -0.779          1.369          0.122      -0.001    -0.123              55.316            9.577

3.00          -1.353          1.878          0.150        0.031    -0.119              56.472            9.670

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00            327.017            5.899            327.017            5.899                0.512          13.268

1.00          1113.212          17.225            196.647            3.936                0.495          13.285

2.00          -692.276          -8.362            142.475            3.076                0.440          13.340

3.00          -247.520          -1.863            112.500            2.577                0.322          13.458

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00            0.866          0.866            0.870          0.870          0.757          0.758        -0.000

1.00            1.298          0.436            0.875          0.866          0.766          0.750        -7.769

2.00            1.733          0.008            0.882          0.862          0.777          0.743      -15.597

3.00            2.172        -0.417            0.890          0.857          0.792          0.735      -23.553

SRF Front w/3 1/8" Ride Height

Ride Travel is    1.000 (Droop or Rebound)          Steering Rack travel is      0.000

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.870          0.870            2.00%          2.16%          Plunge not done

1.00            0.875          0.866            3.20%          1.09%          Plunge not done

2.00            0.882          0.862            4.68%          0.12%          Plunge not done

3.00            0.890          0.857            6.61%        -0.87%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00            0.175          0.175            0.913          0.913

1.00            0.051          0.291            0.974          0.856

2.00          -0.083          0.402            1.041          0.801

3.00          -0.231          0.509            1.115          0.748

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                  0.000

1.00                  0.002

2.00                  0.005

3.00                  0.008

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is -1.000(    1.000 Bump or Jounce)          Steering Rack travel is      0.0

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -5.517        -5.517            -0.000          -1.316          -0.034      -0.034    -0.067

1.00          -6.234        -4.851            -0.713          -1.315          -0.030      -0.037    -0.067

2.00          -7.004        -4.237            -1.485          -1.313          -0.027      -0.040    -0.067

3.00          -7.829        -3.673            -2.379          -1.310          -0.024      -0.043    -0.067

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00            2.460          2.460        -0.062        0.062      0.125              58.100            9.839

1.00            1.949          2.961        -0.032        0.091      0.123              59.357            9.932

2.00            1.428          3.452        -0.001        0.117      0.118              60.651          10.022

3.00            0.895          3.935          0.031        0.140      0.109              62.002          10.108

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00              92.639          -4.296              92.639          -4.296              -1.316          13.096

1.00            111.570          -5.304              79.311          -3.587              -1.315          13.095

2.00            140.977          -6.885              69.334          -3.051              -1.313          13.093

3.00            193.763          -9.762              61.526          -2.623              -1.310          13.090

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00          -0.857        -0.857            0.853          0.852          0.728          0.726          0.000

1.00          -0.434        -1.278            0.858          0.847          0.736          0.717        -7.653

2.00          -0.008        -1.695            0.863          0.840          0.745          0.705      -15.329

3.00            0.420        -2.109            0.867          0.831          0.751          0.690      -23.055

SRF Front w/3 1/8" Ride Height

Ride Travel is -1.000(    1.000 Bump or Jounce)          Steering Rack travel is      0.0

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.853          0.852          -1.90%        -2.08%          Plunge not done

1.00            0.858          0.847          -0.82%        -3.25%          Plunge not done

2.00            0.863          0.840            0.31%        -4.89%          Plunge not done

3.00            0.867          0.831            1.14%        -6.94%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00            0.618          0.618            0.694          0.694

1.00            0.514          0.722            0.746          0.642

2.00            0.406          0.826            0.799          0.591

3.00            0.296          0.931            0.854          0.539

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                  0.000

1.00                  0.001

2.00                  0.002

3.00                  0.004

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is    2.000 (Droop or Rebound)          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -4.663        -4.663              0.000            1.340          -0.039      -0.039    -0.077

1.00          -5.747        -3.652              2.427            1.314          -0.028      -0.048    -0.076

2.00          -6.912        -2.705              5.304            1.227          -0.016      -0.056    -0.072

3.00          -8.170        -1.820              9.319            1.051          -0.002      -0.064    -0.065

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00          -0.779        -0.779          0.123      -0.123    -0.246              50.268            9.124

1.00          -1.343        -0.231          0.150      -0.094    -0.244              51.599            9.252

2.00          -1.925          0.304          0.175      -0.064    -0.239              52.842            9.367

3.00          -2.530          0.828          0.199      -0.033    -0.231              54.022            9.472

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00          -669.825        -31.639          -669.825        -31.639                1.340          13.440

1.00          -244.392        -10.423          1173.173          59.428                1.314          13.466

2.00          -141.970          -5.172            331.800          17.665                1.227          13.553

3.00            -95.253          -2.654            198.317          10.954                1.051          13.729

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00            1.742          1.742            0.883          0.882          0.779          0.778          0.000

1.00            2.180          1.307            0.891          0.876          0.793          0.768        -8.190

2.00            2.624          0.875            0.901          0.871          0.812          0.759      -16.497

3.00            3.074          0.445            0.916          0.867          0.839          0.752      -25.061

SRF Front w/3 1/8" Ride Height

Ride Travel is    2.000 (Droop or Rebound)          Steering Rack travel is      0.000

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.883          0.882            4.94%          4.96%          Plunge not done

1.00            0.891          0.876            6.83%          3.58%          Plunge not done

2.00            0.901          0.871            9.40%          2.40%          Plunge not done

3.00            0.916          0.867          12.99%          1.39%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00          -0.086        -0.086            1.042          1.042

1.00          -0.234          0.049            1.116          0.976

2.00          -0.404          0.173            1.200          0.914

3.00          -0.601          0.289            1.298          0.857

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                  0.000

1.00                  0.004

2.00                  0.008

3.00                  0.014

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is -2.000(    2.000 Bump or Jounce)          Steering Rack travel is      0.0

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -6.246        -6.246            -0.000          -2.247          -0.098      -0.098    -0.195

1.00          -6.859        -5.684            -0.469          -2.245          -0.096      -0.100    -0.195

2.00          -7.524        -5.173            -0.970          -2.236          -0.094      -0.102    -0.196

3.00          -8.240        -4.714            -1.540          -2.221          -0.092      -0.104    -0.196

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00            3.512          3.512        -0.118        0.118      0.235              60.991          10.096

1.00            3.015          4.000        -0.092        0.140      0.232              62.395          10.190

2.00            2.508          4.478        -0.063        0.158      0.221              63.882          10.283

3.00            1.991          4.948        -0.033        0.170      0.203              65.477          10.376

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00              69.128          -5.477              69.128          -5.477              -2.247          13.027

1.00              79.046          -6.358              61.359          -4.777              -2.245          13.025

2.00              92.282          -7.528              55.067          -4.198              -2.236          13.016

3.00            111.052          -9.188              49.835          -3.702              -2.221          13.001

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00          -1.705        -1.705            0.841          0.842          0.708          0.709        -0.000

1.00          -1.288        -2.119            0.849          0.833          0.721          0.694        -7.807

2.00          -0.867        -2.527            0.854          0.822          0.729          0.676      -15.630

3.00          -0.444        -2.930            0.859          0.808          0.739          0.653      -23.485

SRF Front w/3 1/8" Ride Height

Ride Travel is -2.000(    2.000 Bump or Jounce)          Steering Rack travel is      0.0

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.841          0.842          -4.65%        -4.43%          Plunge not done

1.00            0.849          0.833          -2.92%        -6.42%          Plunge not done

2.00            0.854          0.822          -1.79%        -8.89%          Plunge not done

3.00            0.859          0.808          -0.53%      -11.93%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00            0.829          0.829            0.591          0.591

1.00            0.725          0.934            0.642          0.539

2.00            0.621          1.040            0.693          0.486

3.00            0.516          1.150            0.745          0.432

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                -0.000

1.00                  0.001

2.00                  0.003

3.00                  0.004

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is    3.000 (Droop or Rebound)          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -4.916        -4.916              0.000            2.027          -0.098      -0.098    -0.196

1.00          -6.177        -3.750              2.993            1.971          -0.081      -0.112    -0.193

2.00          -7.549        -2.665              6.714            1.781          -0.060      -0.124    -0.185

3.00          -9.061        -1.651            12.502            1.356          -0.035      -0.135    -0.170

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00          -1.911        -1.911          0.176      -0.176    -0.352              47.274            8.849

1.00          -2.504        -1.339          0.199      -0.151    -0.350              48.846            9.007

2.00          -3.124        -0.783          0.221      -0.124    -0.345              50.259            9.146

3.00          -3.780        -0.242          0.242      -0.095    -0.337              51.553            9.269

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00          -140.535        -10.050          -140.535        -10.050                2.027          13.753

1.00            -94.532          -5.945          -240.535        -18.708                1.971          13.809

2.00            -67.637          -3.434          -643.598        -53.032                1.781          13.999

3.00            -49.636          -1.647          1257.652        107.966                1.356          14.424

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00            2.634          2.634            0.903          0.903          0.815          0.816        -0.000

1.00            3.083          2.191            0.918          0.893          0.842          0.797        -9.132

2.00            3.542          1.753            0.938          0.885          0.880          0.783      -18.553

3.00            4.013          1.318            0.967          0.879          0.936          0.772      -28.694

SRF Front w/3 1/8" Ride Height

Ride Travel is    3.000 (Droop or Rebound)          Steering Rack travel is      0.000

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.903          0.903            9.80%          9.99%          Plunge not done

1.00            0.918          0.893          13.42%          7.45%          Plunge not done

2.00            0.938          0.885          18.50%          5.56%          Plunge not done

3.00            0.967          0.879          26.06%          4.12%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00          -0.408        -0.408            1.202          1.202

1.00          -0.606        -0.238            1.300          1.118

2.00          -0.847        -0.089            1.419          1.044

3.00          -1.154          0.046            1.571          0.977

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                -0.000

1.00                  0.007

2.00                  0.016

3.00                  0.026

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Ride Travel is -3.000(    3.000 Bump or Jounce)          Steering Rack travel is      0.0

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Chassis          (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Roll in            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

degrees          Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

0.00          -7.188        -7.188              0.000          -3.160          -0.197      -0.197    -0.394

1.00          -7.697        -6.730            -0.459          -3.155          -0.196      -0.198    -0.394

2.00          -8.258        -6.325            -0.943          -3.138          -0.195      -0.199    -0.394

3.00          -8.870        -5.973            -1.477          -3.109          -0.195      -0.199    -0.394

---- Caster ----        -------- Steer --------          Side View Swing Arm

Chassis              (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Roll in            Left/        Right/          Left/        Right/        Net              Length          Height

degrees          Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

0.00            4.558          4.558        -0.158        0.158      0.317              64.349          10.388

1.00            4.075          5.033        -0.141        0.170      0.311              66.009          10.489

2.00            3.581          5.498        -0.118        0.176      0.294              67.813          10.593

3.00            3.078          5.954        -0.092        0.172      0.265              69.795          10.700

Chassis              --------- Front View Swing Arm ----------            Roll Center Height

Roll in              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

degrees              Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

0.00              54.901          -6.118              54.901          -6.118              -3.160          12.940

1.00              61.159          -6.914              49.694          -5.441              -3.155          12.935

2.00              68.879          -7.882              45.272          -4.850              -3.138          12.918

3.00              78.726          -9.104              41.454          -4.323              -3.109          12.889

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis          - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Roll in            Left/          Right/          Left/          Right/        Left/        Right/        Twist

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

0.00          -2.539        -2.539            0.824          0.824          0.680          0.679        -0.000

1.00          -2.131        -2.941            0.836          0.810          0.698          0.657        -8.051

2.00          -1.717        -3.336            0.845          0.793          0.713          0.629      -16.109

3.00          -1.300        -3.721            0.852          0.771          0.725          0.595      -24.184

SRF Front w/3 1/8" Ride Height

Ride Travel is -3.000(    3.000 Bump or Jounce)          Steering Rack travel is      0.0

Chassis          - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Roll in            Left/          Right/          Left/          Right/        Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

0.00            0.824          0.824          -8.46%        -8.46%          Plunge not done

1.00            0.836          0.810          -5.96%      -11.44%          Plunge not done

2.00            0.845          0.793          -3.94%      -15.22%          Plunge not done

3.00            0.852          0.771          -2.29%      -19.81%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Chassis          (degrees per inch)      (degrees per deg)

Roll in            Left/          Right/          Left/          Right/

degrees          Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

0.00            1.044          1.044            0.486          0.486

1.00            0.937          1.153            0.539          0.433

2.00            0.832          1.265            0.591          0.377

3.00            0.728          1.382            0.642          0.320

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Chassis                  (inches per inch)                            (inches per degree)

0.00                -0.000

1.00                  0.002

2.00                  0.004

3.00                  0.006

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Chassis Roll is    0.000 degrees          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Ride              (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Height            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

3.00          -4.916        -4.916              0.000            2.027          -0.098      -0.098    -0.196

2.00          -4.663        -4.663              0.000            1.340          -0.039      -0.039    -0.077

1.00          -4.707        -4.707              0.000            0.512          -0.006      -0.006    -0.012

0.00          -5.000        -5.000              0.000          -0.387          -0.003      -0.003    -0.007

-1.00          -5.517        -5.517            -0.000          -1.316          -0.034      -0.034    -0.067

-2.00          -6.246        -6.246            -0.000          -2.247          -0.098      -0.098    -0.195

-3.00          -7.188        -7.188              0.000          -3.160          -0.197      -0.197    -0.394

---- Caster ----        -------- Steer --------          Side View Swing Arm

Ride                  (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Height            Left/        Right/          Left/        Right/        Net              Length          Height

Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

3.00          -1.911        -1.911          0.176      -0.176    -0.352              47.274            8.849

2.00          -0.779        -0.779          0.123      -0.123    -0.246              50.268            9.124

1.00            0.320          0.320          0.063      -0.063    -0.127              52.921            9.367

0.00            1.397          1.397          0.000      -0.000    -0.000              55.470            9.601

-1.00            2.460          2.460        -0.062        0.062      0.125              58.100            9.839

-2.00            3.512          3.512        -0.118        0.118      0.235              60.991          10.096

-3.00            4.558          4.558        -0.158        0.158      0.317              64.349          10.388

Ride                  --------- Front View Swing Arm ----------            Roll Center Height

Height              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

3.00          -140.535        -10.050          -140.535        -10.050                2.027          13.753

2.00          -669.825        -31.639          -669.825        -31.639                1.340          13.440

1.00            327.017            5.899            327.017            5.899                0.512          13.268

0.00            141.722          -1.932            141.722          -1.932              -0.387          13.167

-1.00              92.639          -4.296              92.639          -4.296              -1.316          13.096

-2.00              69.128          -5.477              69.128          -5.477              -2.247          13.027

-3.00              54.901          -6.118              54.901          -6.118              -3.160          12.940

SRF Front w/3 1/8" Ride Height

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis Roll is    0.000 degrees          Steering Rack travel is      0.000

Ride              - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Height            Left/          Right/          Left/          Right/        Left/        Right/        Twist

Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

3.00            2.634          2.634            0.903          0.903          0.815          0.816        -0.000

2.00            1.742          1.742            0.883          0.882          0.779          0.778          0.000

1.00            0.866          0.866            0.870          0.870          0.757          0.758        -0.000

0.00            0.000          0.000            0.862          0.861          0.742          0.742        -0.000

-1.00          -0.857        -0.857            0.853          0.852          0.728          0.726          0.000

-2.00          -1.705        -1.705            0.841          0.842          0.708          0.709        -0.000

-3.00          -2.539        -2.539            0.824          0.824          0.680          0.679        -0.000

Ride              - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Height            Left/          Right/          Left/          Right/        Left/          Right/

Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

3.00            0.903          0.903            9.80%          9.99%          Plunge not done

2.00            0.883          0.882            4.94%          4.96%          Plunge not done

1.00            0.870          0.870            2.00%          2.16%          Plunge not done

0.00            0.862          0.861            0.00%          0.00%          Plunge not done

-1.00            0.853          0.852          -1.90%        -2.08%          Plunge not done

-2.00            0.841          0.842          -4.65%        -4.43%          Plunge not done

-3.00            0.824          0.824          -8.46%        -8.46%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Ride              (degrees per inch)      (degrees per deg)

Height            Left/          Right/          Left/          Right/

Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

3.00          -0.408        -0.408            1.201          1.201

2.00          -0.086        -0.086            1.042          1.042

1.00            0.175          0.175            0.914          0.914

0.00            0.404          0.404            0.801          0.801

-1.00            0.618          0.618            0.696          0.696

-2.00            0.829          0.829            0.592          0.592

-3.00            1.044          1.044            0.486          0.486

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Ride                      (inches per inch)                            (inches per degree)

3.00                -0.000

2.00                  0.000

1.00                  0.000

0.00                -0.000

-1.00                  0.000

-2.00                -0.000

-3.00                -0.000

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Chassis Roll is    1.000 degrees          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Ride              (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Height            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

3.00          -6.177        -3.750              2.993            1.971          -0.081      -0.112    -0.193

2.00          -5.747        -3.652              2.427            1.314          -0.028      -0.048    -0.076

1.00          -5.652        -3.823              3.624            0.495            0.001      -0.012    -0.011

0.00          -5.826        -4.229            -2.961          -0.386            0.001      -0.008    -0.007

-1.00          -6.234        -4.851            -0.713          -1.315          -0.030      -0.037    -0.067

-2.00          -6.859        -5.684            -0.469          -2.245          -0.096      -0.100    -0.195

-3.00          -7.697        -6.730            -0.459          -3.155          -0.196      -0.198    -0.394

---- Caster ----        -------- Steer --------          Side View Swing Arm

Ride                  (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Height            Left/        Right/          Left/        Right/        Net              Length          Height

Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

3.00          -2.504        -1.339          0.199      -0.151    -0.350              48.846            9.007

2.00          -1.343        -0.231          0.150      -0.094    -0.244              51.599            9.252

1.00          -0.223          0.850          0.094      -0.032    -0.126              54.139            9.477

0.00            0.872          1.912          0.032        0.031    -0.000              56.666            9.699

-1.00            1.949          2.961        -0.032        0.091      0.123              59.357            9.932

-2.00            3.015          4.000        -0.092        0.140      0.232              62.395          10.190

-3.00            4.075          5.033        -0.141        0.170      0.311              66.009          10.489

Ride                  --------- Front View Swing Arm ----------            Roll Center Height

Height              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

3.00            -94.532          -5.945          -240.535        -18.708                1.971          13.809

2.00          -244.392        -10.423          1173.173          59.428                1.314          13.466

1.00          1113.212          17.225            196.647            3.936                0.495          13.285

0.00            195.195          -2.965            112.033          -1.381              -0.386          13.166

-1.00            111.570          -5.304              79.311          -3.587              -1.315          13.095

-2.00              79.046          -6.358              61.359          -4.777              -2.245          13.025

-3.00              61.159          -6.914              49.694          -5.441              -3.155          12.935

SRF Front w/3 1/8" Ride Height

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis Roll is    1.000 degrees          Steering Rack travel is      0.000

Ride              - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Height            Left/          Right/          Left/          Right/        Left/        Right/        Twist

Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

3.00            3.083          2.191            0.918          0.893          0.842          0.797        -9.132

2.00            2.180          1.307            0.891          0.876          0.793          0.768        -8.190

1.00            1.298          0.436            0.875          0.866          0.766          0.750        -7.769

0.00            0.428        -0.426            0.866          0.857          0.749          0.735        -7.623

-1.00          -0.434        -1.278            0.858          0.847          0.736          0.717        -7.653

-2.00          -1.288        -2.119            0.849          0.833          0.721          0.694        -7.807

-3.00          -2.131        -2.941            0.836          0.810          0.698          0.657        -8.051

Ride              - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Height            Left/          Right/          Left/          Right/        Left/          Right/

Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

3.00            0.918          0.893          13.42%          7.45%          Plunge not done

2.00            0.891          0.876            6.83%          3.58%          Plunge not done

1.00            0.875          0.866            3.20%          1.09%          Plunge not done

0.00            0.866          0.857            0.95%        -0.94%          Plunge not done

-1.00            0.858          0.847          -0.82%        -3.25%          Plunge not done

-2.00            0.849          0.833          -2.92%        -6.42%          Plunge not done

-3.00            0.836          0.810          -5.96%      -11.44%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Ride              (degrees per inch)      (degrees per deg)

Height            Left/          Right/          Left/          Right/

Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

3.00          -0.606        -0.238            1.298          1.117

2.00          -0.234          0.049            1.115          0.976

1.00            0.051          0.291            0.975          0.857

0.00            0.294          0.511            0.856          0.748

-1.00            0.514          0.722            0.747          0.644

-2.00            0.725          0.934            0.643          0.540

-3.00            0.937          1.153            0.539          0.433

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Ride                      (inches per inch)                            (inches per degree)

3.00                  0.007

2.00                  0.004

1.00                  0.002

0.00                  0.001

-1.00                  0.001

-2.00                  0.001

-3.00                  0.002

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Chassis Roll is    2.000 degrees          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Ride              (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Height            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

3.00          -7.549        -2.665              6.714            1.781          -0.060      -0.124    -0.185

2.00          -6.912        -2.705              5.304            1.227          -0.016      -0.056    -0.072

1.00          -6.662        -2.996              8.001            0.440            0.009      -0.018    -0.010

0.00          -6.709        -3.510            -6.030          -0.383            0.006      -0.012    -0.006

-1.00          -7.004        -4.237            -1.485          -1.313          -0.027      -0.040    -0.067

-2.00          -7.524        -5.173            -0.970          -2.236          -0.094      -0.102    -0.196

-3.00          -8.258        -6.325            -0.943          -3.138          -0.195      -0.199    -0.394

---- Caster ----        -------- Steer --------          Side View Swing Arm

Ride                  (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Height            Left/        Right/          Left/        Right/        Net              Length          Height

Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

3.00          -3.124        -0.783          0.221      -0.124    -0.345              50.259            9.146

2.00          -1.925          0.304          0.175      -0.064    -0.239              52.842            9.367

1.00          -0.779          1.369          0.122      -0.001    -0.123              55.316            9.577

0.00            0.334          2.417          0.063        0.062    -0.001              57.861            9.791

-1.00            1.428          3.452        -0.001        0.117      0.118              60.651          10.022

-2.00            2.508          4.478        -0.063        0.158      0.221              63.882          10.283

-3.00            3.581          5.498        -0.118        0.176      0.294              67.813          10.593

Ride                  --------- Front View Swing Arm ----------            Roll Center Height

Height              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

3.00            -67.637          -3.434          -643.598        -53.032                1.781          13.999

2.00          -141.970          -5.172            331.800          17.665                1.227          13.553

1.00          -692.276          -8.362            142.475            3.076                0.440          13.340

0.00            322.916          -5.538              92.960          -1.036              -0.383          13.163

-1.00            140.977          -6.885              69.334          -3.051              -1.313          13.093

-2.00              92.282          -7.528              55.067          -4.198              -2.236          13.016

-3.00              68.879          -7.882              45.272          -4.850              -3.138          12.918

SRF Front w/3 1/8" Ride Height

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis Roll is    2.000 degrees          Steering Rack travel is      0.000

Ride              - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Height            Left/          Right/          Left/          Right/        Left/        Right/        Twist

Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

3.00            3.542          1.753            0.938          0.885          0.880          0.783      -18.553

2.00            2.624          0.875            0.901          0.871          0.812          0.759      -16.497

1.00            1.733          0.008            0.882          0.862          0.777          0.743      -15.597

0.00            0.858        -0.849            0.870          0.853          0.757          0.728      -15.282

-1.00          -0.008        -1.695            0.863          0.840          0.745          0.705      -15.329

-2.00          -0.867        -2.527            0.854          0.822          0.729          0.676      -15.630

-3.00          -1.717        -3.336            0.845          0.793          0.713          0.629      -16.109

Ride              - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Height            Left/          Right/          Left/          Right/        Left/          Right/

Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

3.00            0.938          0.885          18.50%          5.56%          Plunge not done

2.00            0.901          0.871            9.40%          2.40%          Plunge not done

1.00            0.882          0.862            4.68%          0.12%          Plunge not done

0.00            0.870          0.853            1.98%        -1.88%          Plunge not done

-1.00            0.863          0.840            0.31%        -4.89%          Plunge not done

-2.00            0.854          0.822          -1.79%        -8.89%          Plunge not done

-3.00            0.845          0.793          -3.94%      -15.22%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Ride              (degrees per inch)      (degrees per deg)

Height            Left/          Right/          Left/          Right/

Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

3.00          -0.847        -0.089            1.417          1.044

2.00          -0.404          0.173            1.199          0.915

1.00          -0.083          0.402            1.041          0.802

0.00            0.177          0.616            0.913          0.697

-1.00            0.406          0.826            0.800          0.593

-2.00            0.621          1.040            0.694          0.488

-3.00            0.832          1.265            0.591          0.377

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Ride                      (inches per inch)                            (inches per degree)

3.00                  0.016

2.00                  0.008

1.00                  0.005

0.00                  0.003

-1.00                  0.002

-2.00                  0.003

-3.00                  0.004

SRF Front w/3 1/8" Ride Height

Fundamental Analysis of Camber, Steer, Scrub and Roll Center

Chassis Roll is    3.000 degrees          Steering Rack travel is      0.000

---- Camber ----        --- Roll Center ---          ----- Tire Scrub -----

(in degrees)                    (in inches)                    (measured in inches)

Ride              (Pos. leans out)        (Pos. Horiz Loaded)            (Pos. is wide track)

Height            Left/        Right/        Horizontal Vertical            Left/        Right/      Net

Unloaded    Loaded        Position      Position          Unloaded    Loaded    Scrub

------          --------    ------        --------      --------          --------    -----      -----

3.00          -9.061        -1.651            12.502            1.356          -0.035      -0.135    -0.170

2.00          -8.170        -1.820              9.319            1.051          -0.002      -0.064    -0.065

1.00          -7.745        -2.224            14.318            0.322            0.017      -0.024    -0.007

0.00          -7.653        -2.844            -9.286          -0.376            0.011      -0.017    -0.006

-1.00          -7.829        -3.673            -2.379          -1.310          -0.024      -0.043    -0.067

-2.00          -8.240        -4.714            -1.540          -2.221          -0.092      -0.104    -0.196

-3.00          -8.870        -5.973            -1.477          -3.109          -0.195      -0.199    -0.394

---- Caster ----        -------- Steer --------          Side View Swing Arm

Ride                  (in degrees)              (inch Pos. is left)            (Anti-dive, squat)

Height            Left/        Right/          Left/        Right/        Net              Length          Height

Unloaded    Loaded        Unloaded    Loaded Toe-In              of Arm          of IC

------          --------    ------        --------    -----    ------              ------          ------

3.00          -3.780        -0.242          0.242      -0.095    -0.337              51.553            9.269

2.00          -2.530          0.828          0.199      -0.033    -0.231              54.022            9.472

1.00          -1.353          1.878          0.150        0.031    -0.119              56.472            9.670

0.00          -0.216          2.912          0.093        0.090    -0.003              59.072            9.878

-1.00            0.895          3.935          0.031        0.140      0.109              62.002          10.108

-2.00            1.991          4.948        -0.033        0.170      0.203              65.477          10.376

-3.00            3.078          5.954        -0.092        0.172      0.265              69.795          10.700

Ride                  --------- Front View Swing Arm ----------            Roll Center Height

Height              Left/Unloaded Side            Right/Loaded Side            Absolute    Relative

Length          Height              Length          Height            to ground    to C.G.

------              ------          ------              ------          ------              ------          ------

3.00            -49.636          -1.647          1257.652        107.966                1.356          14.424

2.00            -95.253          -2.654            198.317          10.954                1.051          13.729

1.00          -247.520          -1.863            112.500            2.577                0.322          13.458

0.00          1065.434        -20.981              79.548          -0.795              -0.376          13.156

-1.00            193.763          -9.762              61.526          -2.623              -1.310          13.090

-2.00            111.052          -9.188              49.835          -3.702              -2.221          13.001

-3.00              78.726          -9.104              41.454          -4.323              -3.109          12.889

SRF Front w/3 1/8" Ride Height

Spring Travel, Motion Ratio (or Velocity Ratio) and Swaybar Twist

Chassis Roll is    3.000 degrees          Steering Rack travel is      0.000

Ride              - Spring Travel -        - Motion Ratio --      -Ratio Squared--      Swaybar

Height            Left/          Right/          Left/          Right/        Left/        Right/        Twist

Unloaded      Loaded        Unloaded      Loaded      Unloaded    Loaded      degrees

------          --------      ------        --------      ------      --------    ------        ------

3.00            4.013          1.318            0.967          0.879          0.936          0.772      -28.694

2.00            3.074          0.445            0.916          0.867          0.839          0.752      -25.061

1.00            2.172        -0.417            0.890          0.857          0.792          0.735      -23.553

0.00            1.290        -1.269            0.875          0.847          0.766          0.717      -23.015

-1.00            0.420        -2.109            0.867          0.831          0.751          0.690      -23.055

-2.00          -0.444        -2.930            0.859          0.808          0.739          0.653      -23.485

-3.00          -1.300        -3.721            0.852          0.771          0.725          0.595      -24.184

Ride              - Motion Ratio --        -- Rising Rate --      Halfshaft Plunge

Height            Left/          Right/          Left/          Right/        Left/          Right/

Unloaded      Loaded        Unloaded      Loaded      Unloaded      Loaded

------          --------      ------        --------      ------      --------      ------

3.00            0.967          0.879          26.06%          4.12%          Plunge not done

2.00            0.916          0.867          12.99%          1.39%          Plunge not done

1.00            0.890          0.857            6.61%        -0.87%          Plunge not done

0.00            0.875          0.847            3.19%        -3.27%          Plunge not done

-1.00            0.867          0.831            1.14%        -6.94%          Plunge not done

-2.00            0.859          0.808          -0.53%      -11.93%          Plunge not done

-3.00            0.852          0.771          -2.29%      -19.81%          Plunge not done

-- Ride Camber --        -- Roll Camber --

Ride              (degrees per inch)      (degrees per deg)

Height            Left/          Right/          Left/          Right/

Unloaded      Loaded        Unloaded      Loaded

------          --------      ------        --------      ------

3.00          -1.154          0.046            1.568          0.978

2.00          -0.601          0.289            1.296          0.858

1.00          -0.231          0.509            1.114          0.749

0.00            0.054          0.720            0.974          0.646

-1.00            0.296          0.931            0.854          0.542

-2.00            0.516          1.150            0.746          0.434

-3.00            0.728          1.382            0.642          0.320

Theoretical Analysis of Changes based on Instant Centers

------- Ride Scrub -------            ------- Roll Scrub -------

Ride                      (inches per inch)                            (inches per degree)

3.00                  0.026

2.00                  0.013

1.00                  0.008

0.00                  0.005

-1.00                  0.004

-2.00                  0.004

-3.00                  0.006